Institute for Manufacturing

# Report Title in this box

Report Number: MET/## - ## - ##

Company Name

Supervisor: Supervisor’s name

Author Name

Date

The content of this report is confidential between the IfM and “Company Name”, and its contents cannot be disclosed without the permission of the METIIB Project Coordinator and/or METIIB Course Director.

# Contents

Contents	3

Executive Summary	7

1.0	Introduction	8

1.1	Project context and the Global Supply Chain Observatory	8

1.2	Problem statement	8

1.3	Aims and objectives	8

1.4	Report structure	8

2.0	Background and Literature Review	9

2.1	Supply-chain environmental monitoring	9

2.2	Satellite earth observation for environmental screening	10

2.3	Existing tools and platforms	10

3.0	Research Gap and Scope	13

3.1	Limitations of the existing approach	13

3.2	Identified gaps	14

3.3	Project scope and boundaries	15

4.1	Overall approach	17

4.2	Stakeholder engagement	17

4.3	Indicator and dataset review	17

4.4	Iterative design and implementation	17

4.5	Verification and testing strategy	17

5.0	Stakeholder Requirements	18

5.1	User types	18

5.2	Core decisions the tool must support	18

5.3	Requirements summary	20

6.0	Environmental Indicator Evaluation	22

6.1	Evaluation criteria	22

6.2	The three-pillar structure	23

6.3	Air Pollution pillar	23

6.4	Greenhouse Gas pillar	25

6.5	Nature and Land pillar	26

6.6	Indicators retained, demoted, and deferred	27

7.0	Metric and Indicator Design	29

7.1	From raw value to score: shared conventions	29

7.2	Air pillar - anomaly grammar	32

7.3	GHG pillar - absolute-anchored activity grammar	32

7.4	Nature pillar - exposure/change/condition grammar	33

7.5	Combining pillars: the composite	34

7.6	Confidence	34

7.7	Attributability	34

7.8	Graceful degradation (survivor-renormalise, CAMS fallback, strict-None)	35

7.9	Design evolution and rejected approaches	36

7.10	Trend	36

8.0	System Architecture and Tool Implementation	38

8.1	From Google Earth Engine scripts to a Python application	38

8.2	System architecture	38

8.3	The indicator engine	38

8.4	Data sources and integration	38

8.5	Testing and verification	38

9.0	Platform and Visualisation Design	39

9.1	Application workflow and page structure	39

9.2	Screening and prioritisation results	39

9.3	Risk matrix and verbal summaries	39

9.4	Reporting and export	39

9.5	Visual alignment with the parent platform	39

10.0	Case Study	40

10.1	Method	40

10.2	High-signal site	40

10.3	Low-signal site	40

10.4	Discussion of results	40

11.0	Discussion	41

11.1	Achievement against objectives	41

11.2	Limitations	41

11.3	What satellite data can responsibly claim	41

12.0	Conclusions and Recommendations	42

12.1	Conclusions	42

12.2	Recommendations for the GSCO team	42

12.3	Future work	42

References	43

Appendices	47

Appendix B	47

Appendix B.1 — Research questions by gap.	47

Appendix B.2 — Full requirements matrix.	47

Appendix C	47

Appendix C.1 — Indicator evaluation scorecard.	47

Appendix E	47

# Executive Summary

[One page maximum. Aims of the project, main conclusions, action points. Written last.]

# 1.0	Introduction

[Section introduction: one sentence outlining what this chapter covers.]

### 1.1	Project context and the Global Supply Chain Observatory

### 1.2	Problem statement

### 1.3	Aims and objectives

### 1.4	Report structure

# 2.0	Background and Literature Review

[Section introduction: one sentence outlining what this chapter covers.]

### 2.1	Supply-chain environmental monitoring

Modern supply chains separate consumption from impact. A product sold in one region may rely on materials extracted, refined, and assembled across several others, and each stage carries its own pollution, emissions, and land-usage. The organisations with the most leverage to request environmental improvement - large buyers, regulators, and investors - are usually the farthest from the point of production. What closes that distance is transparency: the accurate communication of information about operations and products across the network (Bai and Sarkis, 2020). Greater transparency lets firms identify inefficiencies, detect risks earlier, and build trust with investors, customers, and regulators, which is why it has become central to how supply chains are governed (Zhao et al., 2025).

Transparency has conventionally depended on supplier self-reporting, certifications, and periodic audits; each has known limits. Self-reported data is infrequent and selectively disclosed; certification reaches only a fraction of suppliers; and on-site audits are costly, easily anticipated, and capture a single moment. Indeed, two decades of evidence suggest that audit programmes frequently fail to detect or correct the labour and environmental problems they are meant to tackle (LeBaron, Lister and Dauvergne, 2017). Then, for most supplier sites, decision-makers work with little independent, up-to-date environmental information. The constraint is not a shortage of environmental data, but the absence of timely, comparable, independent evidence tied to specific locations and actors. As environmental monitoring becomes core extension to supply-chain transparency under rising regulatory and stakeholder pressure (Li, 2025), satellite earth observation offers one way to provide that evidence. The European Union's Corporate Sustainability Reporting Directive, illustrates this shift, requiring large companies to disclose environmental impacts across their value chains and not only their own operations (European Commission, 2025). Analysis of satellite data can offer a screening layer that points to where impact is likely, not a verdict on who is responsible.

### 2.2	Satellite earth observation for environmental screening

Satellite earth observation offers a consistent, independent, and repeatable view of conditions on the ground, captured the same way everywhere. A single instrument images the entire planet on a fixed schedule, so a remote mine and a coastal refinery are measured by the same sensor against the same standard. For supply-chain screening, this uniformity matters more than precision at any one site as it makes places comparable.

The technical foundations of satellite imagery - sensor types, spatial and temporal resolution, processing levels, and the major public datasets - were reviewed in detail in the earlier phase of this project (Li, 2024). What matters for the present work is the capability this provides. Modern publicly available constellations measure environmental variables of direct relevance to industrial activity at scales fine enough to resolve individual sites. Several of these instruments revisit the same location daily or near-daily (Veefkind et al., 2012), which is what makes them suitable not just for one-off assessment but for the ongoing monitoring a screening tool requires. The current project builds on that foundation, and its contribution lies not in the raw availability of these data but in how they are selected, combined, and translated into a decision-making tool.

However, these capabilities come with an important caveat. Satellites do not observe environmental contribution directly; they record physical signals from which activity and impact must be inferred. A signal consistent with industrial emission over a facility does not, in itself, identify the responsible source, establish whether any regulatory threshold was exceeded, or assign accountability. Satellite evidence is therefore best treated as a screening signal rather than a determination: it localises probable impact and prioritises sites for scrutiny, while attribution of cause and judgement of compliance remain tasks for follow-up against complementary data. This distinction is the organising principle of the tool developed in this project, and it recurs throughout the chapters that follow.

### 2.3	Existing tools and platforms

Several established platforms already apply satellite and open environmental data to questions of supply-chain sustainability, and they define the state of the art against which this project should be understood. Three are particularly relevant: Global Forest Watch, Trase, and Climate TRACE.

Global Forest Watch (GFW), developed by the World Resources Institute, is the most established. It combines two decades of annual tree-cover-change data with near-real-time deforestation and fire alerts derived from satellite imagery, all freely accessible through an interactive map and API (World Resources Institute, 2024). GFW's own documentation distinguishes "early warning" alert systems, which flag where a disturbance has occurred so that resources can be prioritised, from software built to produce reporting, and is explicit that the alerts are designed for the former rather than the latter (Global Forest Watch, 2024). This is precisely the "screening signal, not determination" logic set out in Section 2.2, and it is the model GFW has validated at scale.

Trase takes a different approach. Founded in 2015 by the Stockholm Environment Institute and Global Canopy, it links downstream actors to upstream production regions by combining material-flow analysis with localised deforestation data to quantify an actor's "deforestation exposure" (Stockholm Environment Institute, 2023; Trase, 2024). Its contribution is the connection between commodity flows and impact - it answers who is exposed to risk through their sourcing, at jurisdictional resolution, rather than what is happening at a specific site now. This trade-to-place structure is precisely the one the parent platform in which this project sits is built on. The Global Supply Chain Observatory itself maps critical-industry supply, tracing the trade routes and supplier nodes underpinning those industries. Where Trase connects actors to production regions, the GSCO platform connects supply-chain routes to specific supplier locations, and the environmental tool developed here is designed to attach to those locations. This two-way connection is the mechanism through which it serves policy makers in particular.

Climate TRACE represents a third approach, focused on emissions rather than land. Launched in 2021, it fuses observations from thhousands of satellites and ground-, air-, and sea-based sensors with machine learning to produce a facility-level global inventory of greenhouse-gas emissions, attributing emissions to individual sources across twenty economic sectors (Climate TRACE, 2024). One feature is directly relevant to the design choices made later in this project; it attaches a per-facility "data confidence" measure to its estimates, acknowledging that satellite-derived attribution carries varying degrees of certainty.

The three platforms differ in their data and their focus, but they share a boundary. Each is excellent at making environmental impact visible, yet each stops at the threshold of the decision itself. They tell a user what is happening and where, but offer limited structured guidance on what to do next: which site to investigate first, how to weigh competing signals, or how confident to be. This is the gap in the current landscape which the present project sets out to address. The next chapter examines how the GSCO environmental tool inherited at the start of this project sat against this backdrop, and where its specific limitations lay.

# 3.0	Research Gap and Scope

This chapter turns from the wider monitoring landscape to the specific tool inherited at the start of the project, examining its limitations, the gaps they expose, and the scope this project adopts accordingly.

### 3.1	Limitations of the existing approach

The environmental tool inherited at the start of this project had been developed in an earlier phase of the GSCO’s development (Li, 2024) within the Google Earth Engine (GEE) Code Editor, written in JavaScript. It performed two analyses within the scope of my project. The first mapped atmospheric concentrations of seven gases and aerosols - NO₂, CO, O₃, SO₂, CH₄, formaldehyde, and the absorbing aerosol index - from the European Space Agency's Sentinel-5P satellite. The second mapped vegetation greenness using the Normalised Difference Vegetation Index (NDVI), extracted from NASA's Terra–MODIS instrument as a sixteen-day composite. A user selected a centre point and a radius, and the tool returned summary statistics over that area together with an animated sequence showing change over time. The tool was conceived as a "pointer" to areas of potential concern rather than an instrument of assessment (Li, 2024).

Several limitations constrained the tool’s usefulness:

- The values it reported were expressed in units incompatible with reporting standards and common jargon. For example, gas concentrations were displayed as vertical column densities in micromoles per square metre - the total quantity of a gas in a column of air above the site. Regulatory air-quality standards, by contrast, are framed as surface concentrations in micrograms per cubic metre averaged over fixed periods (World Health Organization, 2021). A column density is not just a rescaling of surface concentrations but a different physical quantity, so the tool's outputs could not be read against the standards its users work to. Beyond this, the radius input carried no guidance on an appropriate value for the screening’s purpose, and interpreting a result required manually cross-referencing the colour shown on the map against a separate legend. No written interpretation of a site's condition was offered.

- The tool displayed indicators but did not convert them into a decision. It produced no score, offered no means of comparing one site against another, attached no measure of how reliable an observation was, and made no connection to the supply-chain actors that the wider GSCO platform already maps. In this respect it reproduced exactly the boundary identified in Section 2.3 for the external platforms: it made environmental conditions visible without offering structured guidance on what to do about them.

[Figure 3.1: Annotated screenshot of the inherited GEE Code-Editor tool — gas-concentration (or NDVI) map view, showing (a) the raw column-density/scaled-NDVI legend, (b) the centre-and-radius input, (c) the map/legend cross-referencing the user must do manually. Source: adapted from Li (2024). To be inserted by author.]

### 3.2	Identified gaps

The scoping phase of the project summarised these shortcomings, together with those of the broader tool landscape, into four gaps. They are summarised below; the full set of research questions associated with each is reproduced in Appendix XX.

- Indicator coverage and design - the inherited indicator set was scientifically valid but incomplete and poorly aligned with how decisions are actually made. It omitted indicators like carbon dioxide (CO2), the principal greenhouse gas, and particulate matter, the pollutant most directly cited within air-quality regulation. Moreover, it offered no aggregated or summary measure to help a non-specialist weigh several signals at once.

- Usability - even where data were available, they were not translated into a form a non-expert stakeholder could interpret, so a high volume of displayed data did not amount to practical insight.

- Decision-making - the tool was descriptive. It was not linked to specific supply-chain actors or tiers, and it gave no guidance on how a user should respond to what it showed.

- Divide between control and collaboration – the tool’s monitoring approach leans towards surveillance and compliance rather than helping suppliers identify where they might improve.

The scoping phase concluded that these gaps share a single root. The binding constraint on the current GSCO tool was not a lack of environmental data but the absence of integration between the indicators on offer, their usability, and the decisions they were meant to support. It is that integration which this project sets out to provide.

### 3.3	Project scope and boundaries

The project addresses the first three gaps directly and the fourth in a limited form.

In response to the indicator gap, it re-evaluates the inherited indicators and extends the set; rather than presenting raw measurements, expresses each indicator as a comparable score on a common scale, accompanied by a measure of observational confidence that records how well the satellites were able to observe a given site (developed in Section 7.5).

In response to the usability and decision-making gaps, it begins from an analysis of who the tool's stakeholders are (Section 5), and organises the scored indicators into four decisions a user can act on — screening, monitoring, prioritisation, and reporting. These are presented through a user-facing interface built for non-specialists, using a traffic-light summary and plain-language explanations of each result, amongst other features.

The fourth gap is addressed through the reporting function, which renders the results of a screening in a format that is easy to view and export, as well as integrate to the GSCO’s visualisation tools. This is so that a policy maker or a buyer and its suppliers can share a common record and locate the areas where improvement would matter most.

Underpinning all of this, the project migrates the tool from the GEE Code Editor to a standalone Python application, both to support a more substantial analytical pipeline and to allow an interface that can be aligned with the rest of the GSCO platform (the rationale is developed in Section 8.1). The scored indicators are not left as untested assertions; selected indicators were validated against independent reference datasets (Section 10). Finally, attribution - the question of whether an observed signal can be tied to a particular site - is surfaced explicitly in the analysis rather than left implicit.

Two boundaries define what the project does not claim. The first follows directly from the logic established in Section 2.2: the tool produces a screening signal, not a determination. It localises probable impact and ranks sites for scrutiny, but it does not prove that a particular supplier caused an observed signal, nor does it establish whether any regulatory threshold has been breached. Where attribution is computed, it is hence presented as indicative context that informs a judgement. The second boundary concerns the basis of the scores. The indicators are computed from satellite and modelled-atmosphere data, without integration of continuous ground-based instrumentation at each site. This is a deliberate choice to make the tool globally and uniformly applicable. Beyond this, the scoring parameters are set on scientific reasoning and the resulting indicators were checked against independent on-the-ground data (Section 10). Individual thresholds which remain preliminary are noted alongside the relevant indicator and reviewed in Section 11.

Within these boundaries, the chapters that follow set out how the project was carried out, beginning with the methodology in Section 4.
4.0	Methodology

[Section introduction: one sentence outlining what this chapter covers.]

### 4.1	Overall approach

### 4.2	Stakeholder engagement

### 4.3	Indicator and dataset review

### 4.4	Iterative design and implementation

### 4.5	Verification and testing strategy

# 5.0	Stakeholder Requirements

The preceding chapters established that satellite data’s value depends on being translated into decisions. This chapter identifies whose decisions, and what those decisions require. The requirements set out here were derived from stakeholder engagement during the project, principally consultation with the GSCO team. They define the criteria against which environmental indicators are evaluated in Section 6.

### 5.1	User types

The Global Supply Chain Observatory is intended to serve a range of stakeholders, but two groups are prioritised in the design of the environmental tool: policy makers and regulators first, followed by multinational corporations (MNCs). This ordering follows the project brief, which identifies policy makers as GSCO's first target users, while requiring that the platform remain useful across a wider set of actors. Prioritising one group does not therefore narrow the tool to it; it determines whose needs lead the design.

For policy makers, value lies in seeing the system rather than the single site: using the tool to identify environmental hotspots and dependencies across critical supply chains, and to locate the gaps in data or standards where intervention, investment, or stronger reporting requirements are warranted. For MNCs, value is more operational: translating satellite-derived indicators into supplier-level decisions - which sites to screen, which require due diligence, which to monitor more closely, and where to act.

The requirements of MNCs are particularly shaped by a changing regulatory environment. As Section 2.1 noted, mandatory sustainability disclosure increasingly extends across the value chain rather than a company's own operations alone, under frameworks such as the European Union's Corporate Sustainability Reporting Directive and its associated reporting standards (European Commission, 2025). An MNC's interest in the tool is therefore not only operational but evidentiary: its outputs must be capable of supporting due-diligence and ESG disclosure obligations. This requirement is treated as a stakeholder need here and returns as a concrete design choice in the indicator units of Chapter 6 and the reporting design of Chapter 9.

### 5.2	Core decisions the tool must support

The tool is organised around four decisions a user can make about a location or a set of locations - screening, monitoring, prioritisation, and reporting - connected by the workflow shown in Figure 5.1. These are not separate products but stages a user can move between, sharing a common starting point.. A policy maker typically begins from the supply-chain map GSCO already provides, selecting a region or a node from an existing network. An MNC begins from a supply chain it has uploaded itself. Either way, once a location is chosen, the same four decisions and analysis become available.

[Figure 5.1: Tool workflow flowchart — landing and user-type selection → scope set-up → Workflow Hub → Inspect and Prioritisation branches with their result views → save to persistent modules. To be inserted by author.]

Screening answers the most immediate question: does this location show cause for concern now? The user selects a single region or supplier and sets a radius defining the site buffer around it, choosing from a graded set of stops - 1, 5, 10, 25, or 50 km - that range from an individual facility footprint to a broad regional extent; a 5 km buffer is suggested for a single supplier and a 25 km one for a region. The tool characterises conditions within that buffer relative to its wider surroundings, within a customisable time frame; the tool defaults to the most recent 90-day window so that the result reflects current conditions.. What returns is a set of scores with a traffic-light rating and a map of where the signal concentrates, supporting a rapid judgement of whether the site requires closer analysis. This is the tool’s primary role: a first-pass filter pointing to probable impact. The method by which the buffer-versus-surroundings comparison becomes a score is set out in Section 7.

Monitoring assesses whether conditions within the same location are changing over time. Rather than a single snapshot, the tool fits a trend to each indicator's history, estimating the direction and rate of change, together with a measure of whether that change is statistically distinguishable from noise. One point is worth making explicit. The requirement, as first expressed in the Project Brief, asked for a single combined measure of change for a location. The tool instead reports change separately for each environmental indicator it tracks, since different signals change in different ways and are not meaningfully averaged. For the purposes of this chapter, the monitoring features simply idenntifies whether conditions at a site are getting worse.

Prioritisation shifts from one location to many. Rather than inspecting a single site, the user runs the analysis across a set of supply chain nodes, applying the same radius around every node so that the results are comparable. The tool returns the locations ranked by the most and least pressing stand out – according to a pre-defined environmental criteria - allowing limited investigative capacity to be directed where it is most likely to be justified. To keep the satellite computation manageable, a single run covers up to 30 nodes.

Reporting turns any of the preceding analyses into a communicable record. The user assembles the indicators and evidence from a screening, monitoring, or prioritisation run into a structured report - framed for policy audit in the policy-maker case, and for ESG and due-diligence disclosure in the MNC case – and exports it. XXX

Taken together, these four decisions define what the tool must do; the next section consolidates them, with their associated indicators and outputs, into the requirements matrix that guided the design.

### 5.3	Requirements summary

The requirements set out above can be consolidated into a single matrix relating each stakeholder group to each decision it needs to make.

The tool produces the same analytical outputs for both user groups; what changes is scope and labelling, not computation. A screening run over a region and a screening run over a single supplier site use the same underlying engine, and the result is presented to a policy maker as a "Regional" score and to an MNC as the "Supplier" equivalents. The distinctions in the table are therefore distinctions of who is asking and at what scope.

| Decision Stage | Output (policy-maker / MNC label) | What the user is deciding |
| --- | --- | --- |
| Screening | Environmental Screening Score | Whether a location shows cause for concern now |
| Monitoring | Per-indicator environmental trends | Whether conditions at a location are changing over time |
| Prioritisation | Environmental Audit Priority Score | Which sites, among many, to investigate first |
| Reporting | Policy audit / ESG & due-diligence report | How to communicate findings as defensible evidence |

Table 2: Stakeholder decisions and corresponding outputs. The same analytical output is labelled "Regional" for policy makers and "Supplier" for MNCs; the difference is scope and framing, not computation. The full requirements matrix - environmental indicators, visualisations, and report types for each row - is given in Appendix [X].

The  full requirements matrix outlines, for each combination of stakeholder and decision, the environmental indicators drawn on, the decision indicator produced, the visualisation that presents it, and the report it feeds. It represents requirements as elicited during stakeholder engagement rather than the state of the delivered tool which is the subject of Section 6, instead.

Together, the four decisions and the matrix below them set the criteria against which the environmental indicators are evaluated in the next section.

# 6.0	Environmental Indicator Evaluation

This section sets out how candidate indicators were evaluated against the requirements established in Section 5, the three-pillar structure adopted as a result, and the indicators that populate each pillar.

### 6.1	Evaluation criteria

The scoping phase had identified that the difficulty in this domain is not a lack of environmental data but the absence of a structured basis for deciding which indicators are useful (Section 3.2). An indicator can be scientifically sound yet contribute nothing to a procurement or policy decision. Before any indicator was selected, therefore, a formal evaluation scorecard was constructed so that every candidate - whether inherited or newly proposed - was judged against the same criteria. Indicators were then admitted on their contribution to decision-making rather than their novelty.

The scorecard assesses each candidate against ten criteria, which fall into four groups.

- Relevance asks whether the indicator bears on a recognised environmental-reporting concern (e.g. greenhouse gases) and whether it supports one of the four decisions of Section 5.

- Feasibility covers whether the indicator can be accurately computed: its availability in Google Earth Engine, whether the tool can derive it from raw data, and how frequently it updates.

- Trustworthiness covers whether the indicator can be tied to a specific site, whether its uncertainty and attribution risks are understood, and whether it supports comparison across sites and time.

- Communicability asks whether a non-technical user can interpret the output and insert it in a structured report.

An indicator had to perform across all four groups. The full scorecard, with every candidate indicator scored against the ten criteria, is given in Appendix C.

### 6.2	The three-pillar structure

The evaluation organised the selected indicators into three environmental pillars: Air Pollution, Greenhouse Gas (GHG), and Nature and Land. This structure extends the current tool's division between atmospheric and land indicators, but with a refinement: the single atmospheric category is split into Air Pollution and Greenhouse Gas. The two answer different questions; local air pollution concerns the pollutants that affect health and regulatory air quality around a site, whereas greenhouse-gas relevance concerns a site's contribution to climate impact. Equally, they map to distinct strands of environmental disclosure.  In fact, the three pillars align with the principal environmental topics of the European Sustainability Reporting Standards (Section 2.1): pollution (ESRS E2), climate change (ESRS E1), and biodiversity and ecosystems (ESRS E4). Thus, structuring the tool around these impact-areas allows the screening result to map onto the categories its users must already report against.

[Figure 6.1: The three-pillar structure. A simple schematic showing the three pillars — Air Pollution, Greenhouse Gas, Nature and Land — each mapped to its ESRS topic (E2 Pollution, E1 Climate, E4 Biodiversity), the indicators feeding it, and the three pillar follow-up-priority scores converging into a single composite. To be inserted by author; can be drawn from the engine pillar/composite structure.]

The three pillars are scored differently, because they ask physically different questions. Air pollution is assessed based on how far conditions at a site rise above its surroundings. Greenhouse-gas relevance is assessed against absolute markers of “emitting” activity; and nature is assessed as a combination of biodiversity exposure, vegetation change, and condition. The mechanics of each are the subject of Section 7; what matters at this stage is that the pillars are not forced onto a common scale prematurely. Each pillar yields its own follow-up-priority score, and only at the final stage are the three combined into a single composite (Section 7.3), so that a user can read the composite figure or analyse any one pillar without the scores having been implicitly averaged together.

### 6.3	Air Pollution pillar

The Air Pollution pillar draws on the Sentinel-5P satellite's TROPOMI instrument, but reframes its outputs as screening proxies for industrial activity rather than as raw concentrations. Six measures populate the pillar.

Nitrogen dioxide (NO₂) and carbon monoxide (CO) are combustion proxies: NO₂ is a comparatively short-lived, more spatially localised signal of combustion and traffic, while CO is longer-lived and travels regionally, making it weaker for site attribution but useful as corroboration. TROPOMI's resolution is fine enough that NO₂ plumes from individual large sources, such as power plants and industrial complexes, can be distinguished (Liu et al., 2024).

Sulphur dioxide (SO₂) is a proxy for heavy industry - smelting, coal combustion, refining. Satellite SO₂ retrievals have been used to compile global catalogues of exactly these point sources, dominated by power plants, smelters, and oil-and-gas operations (Fioletov et al., 2020), and is thus the most diagnostic of metals and mining activities.

Formaldehyde (HCHO) is a proxy for volatile-organic-compound and photochemical activity.

Ozone (O₃) is included only as regional air-quality context: it is a secondary pollutant not directly emitted by a facility, so its contribution to the score is deliberately capped to prevent it driving a site-level result it cannot legitimately explain.

The sixth measure addresses particulate matter, the pollutant most directly tied to air-quality regulation and absent from the current tool. Particulate matter cannot be measured directly by satellites, so the pillar uses two related aerosol signals as proxies. The Absorbing Aerosol Index (AAI) flags smoke, dust, and ash, and aerosol optical depth (AOD) measures the total aerosol burden in the atmospheric column. Satellite-derived AOD is well established in the literature as a proxy for surface PM₂.₅, the correlation having been demonstrated across many regions and sensor types (Chu et al., 2023), including specifically from Sentinel-5P TROPOMI retrievals (Mamić et al., 2023), although the relationship is sensitive to humidity, boundary-layer height, and aerosol type and so varies geographically (Amiri and Shahne, 2025). For this reason the pillar also incorporates a modelled PM₂.₅ field from the CAMS atmospheric model; because that field is coarse, it is treated as regional context rather than a fenceline measurement, a limitation examined in Section 11.

These six measures combine into interpretable sub-aggregates:

- combustion proxy,

- heavy-industry score,

- particulate/aerosol score,

and then into the pillar's headline air follow-up priority. The pollutants and their proxy roles are summarised in Table 6.1.

[Table 6.1: Air Pollution pillar — for each measure (NO₂, SO₂, CO, HCHO, O₃, PM/aerosol via AAI and AOD/CAMS): the activity it proxies, its principal attribution confounders, and the Earth Engine source dataset. To be drawn from the Final Indicators List "quality/attributability" column. To be inserted by author.]

### 6.4	Greenhouse Gas pillar

For the Greenhouse Gas pillar, the scoping phase identified the complete absence of any CO2 representation as the most important indicator gap; the current tool offered no signal for the principal greenhouse gas, and therefore could say nothing about a site's climate relevance. This, however, is the dimension that mandatory disclosure (ESRS E1, the GHG Protocol's Scope 1–3 framing) demands. The pillar was introduced to fill that gap, with the explicit caveat that it identifies sites where GHG emissions verification should be prioritised, and does not estimate Scope 1, 2, or 3 emissions.

Four signals were evaluated, falling into two kinds. The first kind is a direct measurement of a greenhouse gas itself. Two were considered: a fossil-CO₂ emissions inventory (ODIAC) and satellite methane (Sentinel-5P CH₄), methane being the second most important anthropogenic greenhouse gas after CO₂ (IPCC, 2021) and the one most detectable from orbit. The second kind is indirect - a proxy for the intensity of activity at a site. The reasoning is that GHG emissions are produced by combustion and energy-intensive industrial operations, so an observable marker of such operation is informative about likely emissions even when the gas itself cannot be measured at site scale. Two activity proxies were considered: a night-lights signal (VIIRS) and the combustion proxy from the Air pillar. Night-time light radiance is a well-established proxy for the scope, reflecting intensity of human and economic activity (Gibson, Olivia and Boe-Gibson, 2020). Moreover, the VIIRS sensor specifically detects the intense thermal trace of industrial combustion sources such as gas flaring (Elvidge et al., 2016), allowing it to discern energy-intensive operation from settlement lighting.

The evaluation found that the two most appealing candidates, the CO₂ inventory and direct methane, could not provide a live screening signal. Both were consequently demoted to context; the reasoning is set out in Section 6.6. The pillar therefore incorporates the signals that survived evaluation as live, present-day discriminators of intense activity: a night-lights flaring signal and the borrowed combustion proxy. This is an honest reflection of what the satellite data can presently support, and the CO₂ and methane signals remain visible to the user as standing context alongside the live score.

### 6.5	Nature and Land pillar

The Nature and Land pillar is organised around three audit questions, adopted from the indicator-evaluation framework developed in the scoping phase:

- whether a site sits in or near an ecologically sensitive area,

- whether natural habitat around it has been converted,

- whether the surrounding vegetation is healthy, stressed, or recovering.

Each of the three maps onto a recognised dimension of ESG audit requirements. Proximity to sensitive areas matters because biodiversity impact is inherently location-based: ESRS E4 requires users to disclose the sites it owns in or near protected areas (EFRAG, 2023), and the TNFD's LEAP requires locating where operations intersect biodiversity-important sites as the first step of any assessment (TNFD, 2023). Habitat conversion matters because the clearing of natural or semi-natural land for an open-pit mine, a quarry or a new industrial footprint is among the most direct and irreversible ways an operation degrades an ecosystem; it is thus a primary driver of biodiversity loss that E4 and the TNFD expect to be assessed. A screening tool that could see neither of these would miss the impacts its users should now report.

The current tool addressed only the last of the audit questions through NDVI. NDVI - the Normalised Difference Vegetation Index - measures vegetation greenness from the contrast between near-infrared and red reflectance, and while greenness is a reasonable indicator of vegetation condition, it is a poor proxy for biodiversity or land-use impact on its own. In other words, a site surrounded by dense monoculture plantation or seasonal crops will register high NDVI while sitting inside a critical habitat or on recently converted land. Thus, greenness alone cannot distinguish ecological health from agricultural intensity, nor detect the conversion of natural land to built or bare ground. It was therefore necessary to add measures of exposure (does the supplier pose a risk to ecologically valuable areas?) and change (has the supplier converted land around it?) to sit alongside the inherited measure of condition.

To do so it adds covers three impact-areas. Biodiversity exposure is assessed using a Key Biodiversity Areas (KBA) layer, measuring a site's proximity to or overlap with designated sensitive areas to indicate risk of impact rather than impact itself. The KBA dataset is recognised under ESRS and the TNFD as a definition of "biodiversity-sensitive area," so a proximity flag against it is appropriate (BirdLife International, 2023). Habitat conversion is assessed using Google's Dynamic World near-real-time land-cover product, which classifies the area into land-cover types and so allows the tool to detect conversion of natural land to built-up or bare ground around a site. Vegetation condition is retained from the inherited tool but recast as a site-versus-surroundings anomaly rather than a raw NDVI value. The three strands form sub-aggregates:

- biodiversity exposure,

- habitat conversion,

- vegetation condition

that combine into the pillar's nature follow-up priority. As in the Air pillar, the comparison of a site against its local surroundings is the shared logic underlying the scoring, developed in Section 7.2. The pillar's measures and their datasets are summarised in Table 6.2.

[Table 6.2: Nature and Land pillar — for each strand (biodiversity exposure via KBA; habitat conversion via Dynamic World; vegetation condition via MODIS NDVI): the audit question it answers, what it measures, and the Earth Engine source dataset. To be inserted by author.]

### 6.6	Indicators retained, demoted, and deferred

For a screening tool, being explicit about what is not scored matters to avoid overclaiming. The evaluation therefore sorted indicators into three categories. Retained indicators are computed and contribute to a live score. Demoted indicators are still computed and shown to the user as reference or context, but deliberately excluded from any score. Deferred indicators are recognised as valuable but left for a later development.

Three demotions should be outlined in full, because each reflects a genuine limitation of the underlying data rather than a shortcoming of the tool. Methane (CH₄) was demoted because the effective ground footprint of the Sentinel-5P methane retrieval is far coarser than a screening-size site, so the signal could not reliably resolve site-scale sources; the supporting validation is in Section 10. The fossil-CO₂ inventory (ODIAC) was demoted because its data vintage lags the present by two years, so it cannot represent live conditions, though it remains valuable as standing-exposure context. Annual forest-loss data (Hansen) was demoted from the habitat-conversion score because its cumulative, and thus incompatible with the recent-window logic of the other indicators; it survives as a check on whether observed biodiversity loss is regional rather than site-specific. The full disposition of every evaluated indicator across the three categories is given in Table 6.3, and the deferred set defines part of the future-work agenda in Section 12.3.

[Table 6.3: Indicators retained, demoted, and deferred. Three groups. RETAINED (live, scored): the six Air measures; VIIRS flaring and the borrowed combustion proxy (GHG); KBA biodiversity exposure, Dynamic World habitat conversion, NDVI vegetation condition (Nature). DEMOTED (computed, shown as reference/context, not scored): CH₄ (footprint), ODIAC CO₂ (vintage lag), Hansen forest loss (window semantics), per-indicator trend terms (drill-down only). DEFERRED (v1.x): EVI, FIRMS active-fire/recovery, JRC surface water, wind and sector-match in scoring, per-image QA pass-rates. To be finalised against the engine. To be inserted by author.]

# 7.0	Metric and Indicator Design

Section 6 established what each pillar measures and why. This chapter sets out how a raw satellite observation becomes a comparable 0-1 screening score, how the three pillars are combined into a single result, and how the tool reports confidence and attributability of its findings. Each mechanism is stated as a rule and then shown firing on two real screenings.

Figure 7.B (§7.5, “Combining pillars: the composite”) presents the headline pillar signatures for two contrasting worked-example suppliers.

### 7.1	From raw value to score: shared conventions

Three conventions are common to all three pillars.

The first is that every indicator emits two values (Appendix E §0.1): a raw value in physical units to comply to environmental-reporting standards, and a 0-1 score used inside the composites. The composite formulae only ever operate on score as combining raw values in different units (a column density in µmol m⁻², a habitat loss in hectares) would be meaningless.

The second convention is the “repeatable core”: the six-step process (Appendix E §0.2) that turns any continuous field into a comparable score. For a given indicator the tool computes:

- the site value as the mean over the site buffer;

- the background value as the median over the surrounding ring;

- the absolute anomaly as their difference;

- a normalised deviation;

- the hotspot frequency;

- and a confidence.

The site and background geometries are two concentric shapes around the supplier point - an inner site buffer and an outer background ring - illustrated in Figure XX.

[Figure 7.1: Buffer geometry. Schematic of the site buffer (inner circle, radius r_site) and the background ring (annulus from r_site to r_background) around a supplier point, with the 5:1 default ratio annotated. To be inserted by author.]

The background statistics over the annulus:

Pbackground=median p ∈ ringPp,

( 1 )

σbackground=std p ∈ ringPp

( 2 )

where the ring is the annulus rsite<dp≤rbackground and dp is the geodesic distance of pixel p from the supplier point. The median and not the mean was chosen deliberately, so a neighbouring hotspot in the ring doesn't inflate the background and mask the site. σbackground is the spatial standard deviation over the ring on a given date.

The normalised deviation is the z-score – the spatial anomaly on one date – and the engine computes it per date across the selected window,

Z = Psite- Pbackgroundσbackground

( 3 )

and the hotspot frequency is the fraction of observation dates on which the site is significantly anomalous,

HF = Ndates with Z ≥2Nvalid dates

( 4 )

The threshold Z≥2corresponds to roughly the 95th percentile under Gaussian noise, the standard convention for a significant deviation. Thus, HF makes the signal spatial-temporral.

The third convention is normalisation from a raw value to a score (Appendix E §0.4). A raw value is mapped onto [0, 1] by measuring its deviation from the background in units of background standard deviation, clamped so that a 3σ exceedance saturates the score:
Xscore=clampXsite -Xbackgroundk∙σbg, 0, 1,  k =3.

( 5 )

For indicators where lower is worse - NDVI for example - the numerator is inverted to Xbg-Xsite. Values already on a [0, 1] scale (KBA overlap fraction, hotspot frequency, land-cover class probability) pass through unchanged. This is implemented directly:

def to_score(value, bg_median, bg_std, direction="higher_is_worse", k=NORMALISATION_K): if bg_std <= 0: return None # uniform background: a normalised deviation is undefined raw = (value - bg_median) if direction == "higher_is_worse" else (bg_median - value) return max(0.0, min(1.0, raw / (k * bg_std)))

The single most important reading rule follows from this construction (Appendix E §0.7): a score is a site-versus-region anomaly, not an absolute reading. A supplier in a clean region that stands out locally can score higher than one in a dirty region that blends into an already-polluted background. This is the screening-not-determination principle of Section 3.3 made arithmetic; the raw site value is surfaced separately for any absolute or regulatory interpretation.

Full per-indicator results are given in Appendix X, Table X.1 (the repeatable core on a single indicator).

[Table 7.2: Default buffer radii. For each site radius (1, 5, 10, 25, 50, 100 km): the spatial scale it matches and the rationale, plus the 5:1 background ratio. Condensed from Appendix E §6.2. To be inserted by author.]

### 7.2	Air pillar - anomaly grammar

The Air pillar's grammar is the repeatable core: all eight air indicators run the z-score method. The pillar then builds upward in three layers: first the per-pollutant scores combine into interpretable sub-aggregates; then the sub-aggregates form a severity core; then the core is wrapped with measurement quality.

Layer 1 - sub-aggregates. The per-pollutant scores feed the interpretable groupings of Section 6.3. The two that drive the pillar are the pollution proxy (combining each pollutant) and the spatial-temporal anomaly. The ozone contribution to the proxy is capped at 0.5, because as a secondary pollutant ozone supplies regional context but can’t produce a site-level result alone.

Layer 2 – severity core. The proxy and anomaly combine into the following:

Aircore=0.625 Proxy+0.375 Anomaly

With the pollution proxy as the primary level signal and the anomaly as the consistency signal.

Layer 3 – follow-up. The core is then combined with the measurement-quality term shared by all three pillars:

Airfollow-up=0.80 Aircore+0.20 MeasurementQuality

These steps are equivalent to an effective 0.50 / 0.30 / 0.20 across proxy, anomaly, and quality. The weights are not an empirically fitted optimum; calibration of the exact values is deferred (Section 12).

Full per-indicator Air results are given in Appendix X, Table X.2.

### 7.3	GHG pillar - absolute-anchored activity grammar

The GHG pillar deliberately does not use the anomaly grammar (Appendix E §2.2a). It asks "how much sustained industrial activity is concentrated here?", using absolute levels. It is built in the same three layers as the Air pillar.

Layer 1 – sub-aggregates. The combustion proxy - the NO₂/CO score - and the VIIRS flaring fraction, which is the share of site pixels whose window-mean radiance exceeds ≈ 100 nW cm⁻² sr⁻¹. The combustion proxy carries the larger weight because it is the stronger intensity ranker (Spearman ρ ≈ 0.85 against expected industrial tier) and captures the dim-night-light combustion that flaring misses; flaring is complementary.

Layer 2 - severity core. The two terms form the GHG severity core:

CoreGHG=0.60 Combustionproxy+0.40 VIIRSflaring

Layer 3 - follow-up.

GHGfollow-up=0.80 CoreGHG+0.20 MeasurementQuality

Figure 7.A: the borrowed combustion proxy is blind to flaring, which VIIRS catches; where both grammars agree (Escondida, Carajás) the GHG signal is genuinely low. Divergence is itself the signal. Source: Appendix X sweep.

Full GHG reference/context terms are given in Appendix X, Table X.3.

### 7.4	Nature pillar - exposure/change/condition grammar

The Nature pillar uses a third grammar.

Layer 1 - sub-aggregates. Exposure depends on sensitive land cover present, and proximity to a Key Biodiversity Area; it is largely a vector-and-proportion calculation on [0, 1]. Conversion (natural-to-non-natural land change) is a difference between two dated composites, each conversion fraction clamped against a 10% saturation point so that a typical few-per-cent loss does not read as misleadingly clean (Appendix E §3.2). Condition or vegetation health is instead an inverted NDVI anomaly.

Layer 2 - severity core. The three strands combine into the Nature severity core:

Naturecore=0.375 Exposure+0.375 Conversion+0.250 Condition

The weight ordering follows the audit logic of Section 6.5: exposure and conversion are the two impact-bearing questions and carry equal weight; condition is a supporting signal.

Layer 3 - follow-up.

Naturefollow-up=0.80 Naturecore+0.20 MeasurementQuality

These weights are equivalent to an effective 0.30 / 0.30 / 0.20 / 0.20 across exposure, conversion, condition, and quality (Section 12).

Full per-strand Nature results are given in Appendix X, Table X.4.

### 7.5	Combining pillars: the composite

The three pillar follow-up priorities combine into a single screening score by a flat equal-weight mean (Appendix E §4):

Overall=13 Air+13 GHG+13 Nature

Equal weights are the default. With no user-specific basis for privileging one environmental theme over another at screening stage, equal weighting has been selected.

Figure 7.B: two suppliers with opposite risk signatures — an oil-and-gas field led by its GHG pillar, an iron-ore mine led by Nature — showing the pillars discriminate between fundamentally different harms. Source: Appendix X sweep.

### 7.6	Graceful degradation (survivor-renormalise, CAMS fallback, strict-None)

A tool that quietly fills gaps with defaults is misleading worst. This engine never substitutes a default; a failed indicator is set to None, recorded in a failures list, and handled by one of several mechanisms sharing one philosophy: degrade rather than fabricate.

- Survivor renormalisation (Section 7.5) drops missing indicators and rescales surviving weights to sum to one.

- Strict-None propagation carries a missing input through as None, never a zero, via a weighted sum that refuses a partial result:

def _weighted_sum_strict(payload, weights): total = 0.0 for key, weight in weights.items(): value = payload.get(key) if value is None: return None # never fabricate a partial result from incomplete inputs total += weight * value return total

- Two further fallbacks handle specific physical situations:

  - Cloudy areas: the CAMS particulate fallback. Optical satellite retrievals fail under cloud, thinning valid coverage. Where the modelled particulate field has too little valid coverage in the buffer to trust, the particulate score falls back to the absorbing-aerosol index alone rather than reporting on near-empty data (Section 6.3).

  - Coastal and data-sparse area: the climatology fallback. Where a site's own background ring is too sparse to form a stable local baseline - a coastal buffer half over water, or an area with few valid observations - the engine falls back to a climatological background (a longer-run norm for that location) in place of the in-window ring statistics, so a site on a coastline or in a thin-data region is still scored against a meaningful reference rather than dropped.

Full degradation results are given in Appendix X, Table X.6.

### 7.7	Confidence

The tool computes a second quantity for every indicator: a 0-1 confidence. It records the observational quality of the evidence, leaving most of the interpretive judgement to the score.

Confidence is built in two stages. A raw confidence combines four observational terms, each on [0,1]:

craw=0.30 QA+0.30 Nvalid+0.25 anomaly strength+0.15 spatial context

The four terms ask complementary questions.

- QA is the retrieval-quality of the satellite product, capturing how much an observation can be trusted before any site-specific reasoning.

- Nvalid is the number of usable observations collected divided by the number expected for the window, so a cloud-thinned or sparsely-visited window is penalised for thin evidence rather than averaged.

- Anomaly strength reuses the hotspot frequency, rewarding a signal that recurs across dates over one that appears once and vanishes.

- Spatial context checks that the site buffer spans enough native pixels to be resolved at all, penalising a buffer smaller than the sensor's own footprint.

The raw confidence is then adjusted by two multiplicative factors that down-weight evidence the tool can still use but should trust less:

cfinal=craw⋅mcol→surf⋅ mfallback

The column-to-surface multiplier mcol→surf recognises that a satellite measures a column across the atmosphere, not a ground-level concentration. In fact, satellite instruments cannot observe near-surface concentrations directly and are limited to vertically integrated column densities, which are not directly comparable to in-situ surface measurements (Zhang et al., 2025). This column is a weaker surface proxy for some gases than others; even for NO₂, where the relationship is comparatively strong, annual column and surface values correlate at only about R2=0.66 (Goldberg et al., 2021). Longer-lived, more vertically-distributed gases such as CH₄ and CO are weaker surface proxies still, so the multiplier down-weights them relative to NO₂. The fallback penalty mfallback applies only when the engine has had to recover missing data from a same-period-previous-year or climatological substitute (Section 7.6). This is because recovered evidence is real but indirect, and is multiplied by 0.60 or 0.75 accordingly.

Confidence aggregates upward by the same strict-None discipline as the scores (Section 7.6). The pillar confidences then combine into a single figure by the minimum, as a site is only as well-observed as its worst-observed pillar. This is reflected in the headline confidence:
ccomposite=mincair, cghg, cnature

In the interface, confidence is surfaced as a coloured dot beside each score; any indicator below 0.40 is labelled "Sparse" (developed in  Section 9).

| Site | Air conf | GHG conf | Nature conf | Composite (min) | Dropped indicators | Fallbacks fired |
| --- | --- | --- | --- | --- | --- | --- |
| Norilsk | 0.594 | 0.791 | 0.761 | 0.594 | PM₂.₅, PM₁₀ | AOD, SO₂ (SPPY) |
| Escondida | 0.694 | 0.712 | 0.832 | 0.694 | PM₂.₅, PM₁₀ | none |

Table 7.7: Per-pillar confidence with the binding composite minimum (bold), shown with each site's degradation events. PM₂.₅ and PM₁₀ drop at every screened site (5 km buffer < 44.5 km native pixel); Norilsk is the only site to fire the SPPY temporal fallback, recovering SO₂ and AOD from polar-winter sparsity.

### 7.8	Attributability

Attributability is a separate, per-indicator categorical signal: high, moderate, low, sparse. It asks what confidence does not: is the observed change actually the supplier's? It is per-indicator rather than per-pillar because attribution physics differ by indicator.

It is deliberately kept external to the composite and confidence chain (Appendix E §2.4), surfacing on the map and in the report so the user can contextualise a finding. Since this is the one place the tool comes close to a causal claim, it is quarantined from every score. Three attributability mechanisms exist, each matched to the physics of its indicators.:

- GHG’s VIIRS lit-contrast classifies the site's brightness as a percentile within the background ring's full pixel distribution (≥0.90→attrbutability is high).

- The Nature habitat spatial link measures the geodesic offset between the supplier point and the centroid of the natural-to-non-natural change pixels (≤1 km →high attributability, ≤ 3km →moderate attributability, beyond → low, <10 pixels →sparse)

- Wind attribution applies only to a specific subset of air pollutants for a physical reason. A satellite sees a pollutant column; whether that column over the site was actually emitted there depends on how far the air moved between emission and overpass. For short-lived gases (atmospheric lifetime of hours) the wind at overpass dominates this, so two multiplicative dampening factors are applied: a speed factor and a direction factor. The speed factor follows Gaussian-plume physics, where concentration scales inversely with wind speed, C∝1/u (Pasquill, 1961; Turner, 1970):

fspeed=urefuref+u,  uref=2 m s-1

The reference speed is the Pasquill calm-to-light wind threshold that underpins Gaussian dispersion modelling (Pasquill, 1961; Gifford, 1961).  Below it, air leaves confidence intact f=1 and strong wind dampens it (at uref,f=0.5). The direction factor compares the upwind and downwind halves of the background ring. If the upwind side is much brighter, an external source is likely feeding the site, and confidence is dampened toward a floor of 0.5:

fdir=clamp​21+r, 0.5, 1.0,  r=bgupwindbgdownwind

Wind is applied to NO₂, SO₂, HCHO, and AAI only as these are the short-lived signals whose attribution today's wind genuinely governs. Below a five-anomaly-day minimum the factors fall back to 1.0, recorded with a low_anomaly_sample flag, so a thin sample never skews a confident attribution.

Full attributability results are given in Appendix X, Table X.5.

### 7.9	Design evolution and rejected approaches

The most consequential correction concerns the denominator of the anomaly z-score (§7.1). In an earlier version the normalising spread σ was the background ring's spatial standard deviation. This inflated anomalies severely, by factors of roughly two to fourteen, because a quiet ring made any site look anomalous. The fix changed the denominator to the site's own temporal day-to-day spread over a trailing climatology baseline - how much the site varies through time - which is physically the right reference. Night-lights are exempted from this temporal denominator, because a persistently-lit source suggests intense industrial activity and yet has near-zero variance.

The GHG’s flaring signal was originally scored by how much brighter the site was than its surroundings. It was abandoned because it could not rank intensity: a modest source and an extreme one both registered as "brighter than background". It was replaced by the absolute-anchor band of Section 7.3, which measures the fraction of site pixels exceeding a fixed brightness threshold and so preserves the discriminatory ordering.

### 7.10	Trend

The trend functions determines whether a signal is changing over time. It is computed per indicator as:

- a Theil–Sen slope - the median of all pairwise slopes across the time series

- a significance. This is the Mann–Kendall two-sided p-value for a monotonic trend, summarised into a bucket that outputs "significant"` when p falls below 0.05.

- a display severity on 0-1, which is the slope expressed in background-σ-per-year and clamped. It is also directional so that the “worrying” sign depends on the indicator;

- a confidence.

Whether a trend is reported at all depends on data sufficiency, while the underlying series and are still displayed so the user can see why.  Both non-parametric trend estimators are chosen over ordinary least-squares (OLS) regression because satellite time series are heavy-tailed. This means that cloud-affected days, retrieval dropouts, and the occasional extreme value would drag an OLS line, but not the median-of-slopes nor the Mann–Kendall p-value, which guards against misreading noise not a trend.

A trend “value” is computed on demand, per indicator; it is never aggregated across indicators, because averaging the incommensurable, noisy, sparse slopes of chemically unrelated signals produces a number with no physical meaning (Section 7.9). Then, it does not the composite or the confidence chain given that it is a monitoring finding, not a screening one. Further details will be provided in the monitoring workflow is described (Section 9).

# 8.0	System Architecture and Tool Implementation

[Section introduction: one sentence outlining what this chapter covers.]

### 8.1	From Google Earth Engine scripts to a Python application

### 8.2	System architecture

### 8.3	The indicator engine

### 8.4	Data sources and integration

### 8.5	Testing and verification

# 9.0	Platform and Visualisation Design

[Section introduction: one sentence outlining what this chapter covers.]

### 9.1	Application workflow and page structure

### 9.2	Screening and prioritisation results

### 9.3	Risk matrix and verbal summaries

### 9.4	Reporting and export

### 9.5	Visual alignment with the parent platform

# 10.0	Case Study

[Section introduction: one sentence outlining what this chapter covers.]

### 10.1	Method

### 10.2	High-signal site

### 10.3	Low-signal site

### 10.4	Discussion of results

# 11.0	Discussion

[Section introduction: one sentence outlining what this chapter covers.]

### 11.1	Achievement against objectives

### 11.2	Limitations

### 11.3	What satellite data can responsibly claim

# 12.0	Conclusions and Recommendations

[Section introduction: one sentence outlining what this chapter covers.]

### 12.1	Conclusions

### 12.2	Recommendations for the GSCO team

### 12.3	Future work

# References

[Reference list. Use the template Reference style. Citation style to be confirmed with supervisor.]

Bai, C. and Sarkis, J. (2020) 'A supply chain transparency and sustainability framework', Journal of Cleaner Production, 246, 118–124. 
doi: https://doi.org/10.1080/00207543.2019.1708989.

LeBaron, G., Lister, J. and Dauvergne, P. (2017) 'Governing global supply chain sustainability through the ethical audit regime', Globalizations, 14(6), pp. 958–975.
doi: https://doi.org/10.1080/14747731.2017.1304008.

Zhao, Y., Wu, W., Shen, Z. and Ji, Z. (2025). Supply chain transparency and firm performance: the role of customer trust. Finance Research Letters, [online] 85, p.107880. 
doi: https://doi.org/10.1016/j.frl.2025.107880.

Li, D. (2025). Supply chain digitalization, corporate governance, and enterprise carbon emissions. International Review of Financial Analysis, 102(104115), p.104115. 
doi: https://doi.org/10.1016/j.irfa.2025.104115.

Li, E. (2024). Enhance Critical Mineral Supply Chain Visibility with Satellite Image. Unpublished report, Institute for Manufacturing, University of Cambridge.

Veefkind, J.P., et al. (2012) 'TROPOMI on the ESA Sentinel-5 Precursor: A GMES mission for global observations of the atmospheric composition for climate, air quality and ozone layer applications', Remote Sensing of Environment, 120, pp. 70–83.
doi: https://doi.org/10.1016/j.rse.2011.09.027.

European Commission (2025) Corporate sustainability reporting. Available at: https://finance.ec.europa.eu/financial-markets/company-reporting-and-auditing/company-reporting/corporate-sustainability-reporting_en (Accessed: 1 May 2026).

Global Forest Watch (2024) Early warning systems for deforestation: an explainer. Available at: https://www.globalforestwatch.org/blog/data-and-tools/early-warning-systems-for-deforestation-an-explainer/  (Accessed: 2 May 2026).

Stockholm Environment Institute (2023) Trase: bringing radical transparency to global supply chains. Available at: https://www.sei.org/features/supply-chain-transparency-trase/  (Accessed: 2 May 2026).

Trase (2024) Methodology. Available at: https://trase.earth/methodology (Accessed: 2 May 2026).

World Resources Institute (2024) Global Forest Watch. Available at: https://www.globalforestwatch.org/ (Accessed: 2 May 2026).

Climate TRACE (2024) About the coalition. Available at: https://climatetrace.org/about (Accessed: 2 May 2026).

World Health Organization (2021) WHO global air quality guidelines: particulate matter (PM2.5 and PM10), ozone, nitrogen dioxide, sulfur dioxide and carbon monoxide. Geneva: World Health Organization. Available at:

https://www.who.int/publications/i/item/9789240034228 (Accessed: 3 May 2026).

Zhu, S., Tang, J., Zhou, X., Li, P., Liu, Z., Zhang, C., Zou, Z., Li, T. and Peng, C. (2023). Research progress, challenges, and prospects of PM2.5 concentration estimation using satellite data. Environmental Reviews, 31(4), pp.605–631. 
doi: https://doi.org/10.1139/er-2022-0125.

Luka Mamić, Mateo Gašparović and Kaplan, G. (2023). Developing PM2.5 and PM10 prediction models on a national and regional scale using open-source remote sensing data. Environmental monitoring and assessment, 195(6). 
doi: https://doi.org/10.1007/s10661-023-11212-x.

Amiri, Z. and Shahne, M.Z. (2025). Modeling PM2.5 Concentration in Tehran Using Satellite-Based Aerosol Optical Depth (AOD) and Machine Learning: Assessing Input Contributions and Prediction Accuracy. Remote Sensing Applications Society and Environment, 38(101549), pp.101549–101549. 
doi: https://doi.org/10.1016/j.rsase.2025.101549.

IPCC (2023). Climate Change 2021 – The Physical Science Basis. Cambridge University Press.

Fioletov, V., McLinden, C.A., Griffin, D., Theys, N., Loyola, D.G., Hedelt, P., Krotkov, N.A. and Li, C. (2020). Anthropogenic and volcanic point source SO2 emissions derived from TROPOMI on board Sentinel-5 Precursor: first results. Atmospheric Chemistry and Physics, 20(9), pp.5591–5607. 
doi: https://doi.org/10.5194/acp-20-5591-2020.

Liu, F., Beirle, S., Joiner, J., Choi, S., Tao, Z., Knowland, K.E., Smith, S.J., Tong, D.Q., Ma, S., Fasnacht, Z.T. and Wagner, T. (2024). High-resolution mapping of nitrogen oxide emissions in large US cities from TROPOMI retrievals of tropospheric nitrogen dioxide columns. Atmospheric Chemistry and Physics, 24(6), pp.3717–3728.
doi: https://doi.org/10.5194/acp-24-3717-2024.

Gibson, J., Olivia, S. and Boe‐Gibson, G. (2020). NIGHT LIGHTS IN ECONOMICS: SOURCES AND USES1. Journal of Economic Surveys, 34(5), pp.955–980.
doi: https://doi.org/10.1111/joes.12387.

Elvidge, C., Zhizhin, M., Baugh, K., Hsu, F.-C. and Ghosh, T. (2015). Methods for Global Survey of Natural Gas Flaring from Visible Infrared Imaging Radiometer Suite Data. Energies, 9(1), p.14. 
doi: https://doi.org/10.3390/en9010014.

Brown, C.F., Brumby, S.P., Guzder-Williams, B., Birch, T., Hyde, S.B., Mazzariello, J., Czerwinski, W., Pasquarella, V.J., Haertel, R., Ilyushchenko, S., Schwehr, K., Weisse, M., Stolle, F., Hanson, C., Guinan, O., Moore, R. and Tait, A.M. (2022). Dynamic World, Near real-time global 10 m land use land cover mapping. Scientific Data, 9(1).
doi: https://doi.org/10.1038/s41597-022-01307-4.

EFRAG (2023) ESRS E4 Biodiversity and Ecosystems. European Sustainability Reporting Standards, Commission Delegated Regulation (EU) 2023/2772.

TNFD (2023) Recommendations of the Taskforce on Nature-related Financial Disclosures. — LEAP "Locate" phase.

BirdLife International (2023) Key Biodiversity Areas / KBA Partnership.

Wang, P., Zeng, G., Gan, R., Wang, J. and Hongbin Zha (2013). Structure-Sensitive Superpixels via Geodesic Distance. International Journal of Computer Vision, 103(1), pp.1–21. 
doi: https://doi.org/10.1007/s11263-012-0588-6.

Pasquill, F. (1961) 'The estimation of the dispersion of windborne material', Meteorological Magazine, 90(1063), pp. 33–49.

Gifford, F.A. (1961) 'Use of routine meteorological observations for estimating atmospheric dispersion', Nuclear Safety, 2(4), pp. 47–51.

Goldberg, D.L., Anenberg, S.C., Kerr, G.H., Mohegh, A., Lu, Z. and Streets, D.G. (2021). TROPOMI NO 2 in the United States: A Detailed Look at the Annual Averages, Weekly Cycles, Effects of Temperature, and Correlation With Surface NO 2 Concentrations. Earth’s Future, 9(4). 
doi: https://doi.org/10.1029/2020ef001665.

Chang, B., Liu, H., Zhang, C. et al. Relating satellite NO2 tropospheric columns to near-surface concentrations: implications from ground-based MAX-DOAS NO2 vertical profile observations. npj Clim Atmos Sci 8, 1 (2025). 
doi: https://doi.org/10.1038/s41612-024-00891-z.

# Appendices

[Appendix A: Development milestones. Appendix B: Full indicator list. Others as needed.]

## Appendix B

Appendix B.1 — Research questions by gap.

[Placeholder: reproduce verbatim the numbered research questions under each of the four gaps from the scoping brief (Radice Fossati, 2026) — Gap 1: Indicator Coverage & Design (Q1–3); Gap 2: Usability (Q4–6); Gap 3: Decision-Making (Q7–9); Gap 4: Control vs Collaboration (Q1–3). I have the full text and will format it as a table when you confirm.]

Appendix B.2 — Full requirements matrix.

## Appendix C

Appendix C.1 — Indicator evaluation scorecard.

[Placeholder: full table, every candidate indicator (rows) × the ten criteria of Section 6.1 (columns), with the score/verdict for each. Source: Final_Indicators_List "Refined Indicator Usefulness Criteria" + per-indicator entries. I have the full content and will format when you confirm the layout.]

## Appendix E

## Appendix X: Worked-example screening data

The tables below give the full per-indicator screening detail underlying Section 7. All values come from the single-supplier worked-example sweep (5 km site buffer, engine-default 5×-capped background ring, latest-valid 90-day window, SPPY temporal fallback on, all 19 indicators).

| Step | Value |
| --- | --- |
| Raw site value (NO₂, ring units) | 79.208 |
| Background median (ring) | 57.383 |
| Anomaly (site − median) | 21.826 |
| Background σ (standardising) | 17.037 |
| z-score (anomaly ÷ σ) | 1.281 |
| Normalised score (0–1) | 0.427 |

Table X.1: The repeatable-core anomaly trace on a single indicator (Escondida, NO₂). Each step is verifiable: site − median = anomaly; anomaly ÷ σ = z; z maps to the normalised score. σ is the standardising (climatology-baseline) spread.

| Indicator | Esc. z | Esc. score | Com. z | Com. score |
| --- | --- | --- | --- | --- |
| NO₂ | 1.281 | 0.427 | 0.362 | 0.121 |
| SO₂ | -0.080 | 0.000 | 0.141 | 0.047 |
| CO | 0.063 | 0.021 | 0.195 | 0.065 |
| HCHO | 0.207 | 0.069 | -0.060 | 0.000 |
| O₃ | -0.023 | 0.000 | 0.031 | 0.010 |
| AAI | 1.055 | 0.352 | -0.041 | 0.000 |
| AOD | -0.127 | 0.000 | 0.464 | 0.155 |

Table X.2: Per-indicator Air results (z and normalised score) — Escondida vs Comodoro Rivadavia. PM₂.₅ and PM₁₀ are omitted (dropped: 5 km buffer < 44.5 km native pixel). Cells shown as — had no valid retrieval in-window.

| Site | CO₂ raw mean | CO₂ rel. int. | VIIRS bright. | CH₄ z | CH₄ score |
| --- | --- | --- | --- | --- | --- |
| Jamnagar | 18810.0 | 5.0 | 61.021 | -0.009 | 0.000 |
| Escondida | 2304.6 | 10.0 | 5.406 | 0.872 | 0.291 |

Table X.3: GHG reference/context terms (displayed, NOT scored) — Jamnagar vs Escondida. CO₂ (ODIAC) relative intensity is capped at 10; CH₄ is an M-CH4-A1 reference signal. None enter the GHG core (combustion + VIIRS flaring).

| Site | KBA overlap (ha) | KBA dist (km) | Proximity branch | Habitat conv. | Nature follow-up |
| --- | --- | --- | --- | --- | --- |
| Carajás | 7,760 | 0.00 | overlap | 0.201 | 0.601 |
| Morowali IMIP | 0 | 5.97 | distance-decay | 0.726 | 0.723 |

Table X.4: Per-strand Nature results — Carajás (inside a KBA: 98.8 % overlap → overlap branch) vs Morowali IMIP (KBA ~6 km away → distance-decay branch; highest on-site habitat conversion). Proximity = max(overlap_pct, exp(−dist/10)).

| Site | VIIRS lit-contrast (state · pct) | Habitat spatial-link (state · offset km) | Wind states |
| --- | --- | --- | --- |
| Morowali IMIP | high · 98.4% | moderate · 1.05 km | aai=sparse, aod=sparse, hcho=sparse, no2=high, so2=high |
| Comodoro (Patagonia) | high · 94.3% | moderate · 2.01 km | aai=sparse, aod=low, hcho=sparse, no2=low, so2=sparse |
| Jamnagar | high · 97.5% | moderate · 1.41 km | aai=low, aod=sparse, hcho=sparse, no2=sparse, so2=sparse |
| Leshan (LONGi) | high · 90.7% | high · 0.13 km | aai=moderate, aod=sparse, hcho=sparse, no2=sparse, so2=sparse |

Table X.5: Attributability labels at the flaring sites and the polysilicon site — VIIRS lit-contrast state (ring percentile), habitat spatial-link state (change-centroid offset from the supplier), and per-pollutant wind states. 'sparse' offsets show as — (too few change pixels to localise).

| Site | Dropped indicators | Fallbacks fired | Survivors (Air) | Composite conf. |
| --- | --- | --- | --- | --- |
| Norilsk | PM₂.₅, PM₁₀ | AOD, SO₂ (SPPY) | 7 of 9 | 0.594 |
| Escondida | PM₂.₅, PM₁₀ | none | 7 of 9 | 0.694 |

Table X.6: Degradation events (full) — Norilsk vs Escondida. PM₂.₅ and PM₁₀ drop at every site (5 km buffer < 44.5 km native pixel), renormalising Air to 7 of 9 survivors. Norilsk additionally recovered SO₂ and AOD via the SPPY temporal fallback; Escondida fired none.

Screening, monitoring, prioritisation, reporting.

the length of the shortest path linking them, where the path is strictly constrained to travel through a specific subset of the image domain (Wang et al., 2013).